ZEBA ACADEMY / INSTRUCTOR – SUFYAN BIN UZAYR

Advanced Parallel Programming & Architecture

Laboratory Examination & Silicon Profiling Worksheet: Empirical Microarchitectural Bottlenecks

Session Duration: 120 Minutes | Total Evaluation: 100 Points | Mode: Individual Bare-Metal Execution

| Student & Hardware Metadata Field | Laboratory Session Specifications & Target Machine Declaration |
| --- | --- |
| Student Full Name | Praliyeva Nuray |
| Student Identification Number | 230103057 |
| Date & Laboratory Session | 17.09.2026 2-N |
| Workstation / Laptop Make & Model |  |
| Processor SKU / SoC Model | 12th Gen Intel(R) Core(TM) i3-1215U |
| Core Topology (Physical P-cores / E-cores) | Physical Cores:6 ¦ Logical SMT Threads: 8 |
| Cache Hierarchy Capacity | L1: KB ¦ L2: 4.5 MB ¦ L3: 10 MB |
| Host OS & Python Runtime | Windows 11, Python 3.12.0 |

- MANDATORY LABORATORY PROTOCOL & HARDWARE FIDELITY NOTICE

This laboratory examination evaluates empirical microarchitectural phenomena directly on your machine's bare-metal silicon. LLMs and AI coding assistants cannot predict the real runtime numbers, thermal throttling curves, or cache eviction latencies of your specific processor SKU. Any submission containing fabricated, perfectly theoretical, or non-empirical data will receive an automatic zero. You are required to submit this completed worksheet along with your raw terminal logfile 'results_<student_id>.txt' before the 120-minute timer elapses.

TASK 1: AMDAHL'S LAW & PHYSICAL SILICON SATURATION

Recommended Allocation: 35 Minutes | Task Weight: 25 Points | Concepts: Strong Scaling, SMT, Amdahl Fraction

Amdahl's Law defines the theoretical upper bound of speedup for a fixed-size workload when scaling across multiple processors:

S(p) = 1 / [ (1 - P) + (P / p) ]

where S(p) is the speedup ratio T(1)/T(p), P is the strictly parallelizable fraction of the algorithm, and p is the number of active worker cores. In physical silicon, speedup curves diverge sharply from theoretical models due to process initialization overhead (fork/spawn latency), inter-process communication (IPC) serialization, OS context switching, and resource contention on Simultaneous Multithreading (SMT/Hyper-Threading) logical cores.

- EXECUTION PROTOCOL

Create a script named 'task1_amdahl.py' using the benchmark code below. Execute the script from your terminal: 'python3 task1_amdahl.py'. Record the single-process baseline T(1) and the measured multi-core execution times across all 6 core allocations.

import time import os

import multiprocessing as mp

def cpu_intensive(chunk_size):

acc = 0

for i in range(chunk_size):

acc += (i % 7) * (i % 11) return acc

def run_bench(workers, total_items=40_000_000): chunk = total_items // workers

start = time.perf_counter()

with mp.Pool(processes=workers) as pool: pool.map(cpu_intensive, [chunk] * workers)

return time.perf_counter() - start

if  name  == " main ":

cores = [1, 2, 4, 8, 12, 16]

base = run_bench(1)

print(f"OS Reported Logical Cores: {os.cpu_count()}") print(f"Single Process Baseline Time T(1): {base:.4f}s\n") print("Cores | Time(s) | Observed Speedup | Theoretical Linear") print("-" * 55)

for c in cores:

t = run_bench(c) speedup = base / t

print(f"{c:5d} | {t:7.4f} | {speedup:16.2f}x | {c:18.2f}x")

| Worker Cores (p) | Measured Time T(p) [s] | Observed Speedup S(p) | Ideal Linear Speedup | Parallel Efficiency [S(p)/p * 100] |
| --- | --- | --- | --- | --- |
| p = 1 (Baseline) | 4.0815 s | 1.00x | 1.00x | 100.0% |
| p = 2 | 2.0636 s | 1.98x | 2.00x | 98.9% |
| p = 4 | 1.3711 s | 2.98x | 4.00x | 74.4% |
| p = 8 | 1.1281 s | 3.62x | 8.00x | 45.2% |
| p = 12 | 1.1396 s | 3.58x | 12.00x | 29.8% |
| p = 16 | 1.2852 s | 3.18x | 16.00x | 19.8& |

Student Response & Architectural Justification:

[Q1.1] Identify the exact inflection point (core count p*) where your observed speedup curve flattens or degrades. Correlate this directly to your machine's physical hardware topology (distinguish between physical Performance cores, Efficiency cores, and logical SMT hyperthreads). (8 Points)

The observed inflection point occurs at approximately p = 8 workers*. Execution time decreases from 4.0815 seconds at 1 worker to 1.1281 seconds at 8 workers, which is the fastest measured result. Increasing the number of workers beyond 8 does not improve performance: the time increases slightly to 1.1396 seconds at 12 workers and to 1.2852 seconds at 16 workers.

The host CPU is a 12th Gen Intel Core i3-1215U with 6 physical cores and 8 logical processors. Its hybrid architecture contains 2 Performance (P) cores and 4 Efficiency (E) cores. The 2 P-cores support Hyper-Threading, producing 8 logical processors in total. Therefore, the empirical saturation point at 8 workers corresponds to the maximum number of logical CPU threads available to the operating system. Beyond this point, additional workers introduce scheduling, context-switching, and CPU resource contention overhead, causing performance to degrade.

Student Response & Architectural Justification:

Using the measured values T(1) = 4.0815 s and T(4) = 1.3711 s:

S(4) = T(1) / T(4)

S(4) = 4.0815 / 1.3711 = 2.9776 ≈ 2.98

According to Amdahl's Law:

S(4) = 1 / [(1 − P) + P/4]

Therefore:

2.9776 = 1 / [(1 − P) + P/4]

1 / 2.9776 = 1 − P + P/4

0.33584 = 1 − 0.75P

0.75P = 0.66416

P = 0.8855

Therefore, the strictly parallelizable fraction of the workload is approximately 88.55%, while the serial fraction is approximately 11.45%.

[Q1.2] Using your empirical measurements for T(1) and T(4), calculate the exact value of P (the parallel fraction) of this workload on your silicon. Show the complete algebraic derivation: S(4) = T(1)/T(4) = 1 / [(1 - P) + P/4]. (9 Points)

Student Response & Architectural Justification:

Using P = 0.8855:

S_max = 1 / (1 − P)

S_max = 1 / (1 − 0.8855)

S_max = 1 / 0.1145

S_max ≈ 8.73×

This means that even an ideal machine with an unlimited number of processors could theoretically achieve no more than approximately 8.73× speedup for this fixed workload. Therefore, purchasing a 128-core server would not produce 128× speedup for this particular application. The serial portion of approximately 11.45% creates an Amdahl's Law limit. In a real system, the actual speedup would be lower because of process-management overhead, communication, scheduling, cache contention, and memory-system limitations.

[Q1.3] Compute the Asymptotic Speedup Limit S_max = lim(p->inf) S(p) = 1 / (1 - P) using your calculated P from Q1.2. Explain what this limit implies if your department purchased a 128-core server node to run this specific workload. (8 Points)

TASK 2: CACHE COHERENCE & THE FALSE SHARING TRAP

Recommended Allocation: 30 Minutes | Task Weight: 25 Points | Concepts: MESI Protocol, 64-Byte Cache Lines, Bus Invalidation

Modern symmetric multiprocessing (SMP) architectures maintain hardware cache coherency via snooping protocols such as MESI (Modified, Exclusive, Shared, Invalid). Data moves between main memory (DRAM) and CPU caches in discrete 64-byte blocks called cache lines. False Sharing occurs when two independent threads executing on physically separate CPU cores continuously write to distinct variables that happen to reside on the same 64-byte cache line. Although no logical data race exists, each write forces the core's cache controller to broadcast an Invalidation signal across the interconnect bus, evicting the line from the other core's L1/L2 cache and causing massive pipeline stalls.

- EXECUTION PROTOCOL

Create 'task2_falsesharing.py'. Run the benchmark 3 separate times to eliminate background OS scheduling noise. The test compares 4 threads writing to adjacent list indices vs. 4 threads writing to padded indices separated by 16 elements (16 * 8 bytes = 128 bytes, guaranteeing placement on distinct 64-byte cache lines). Record the median execution times.

import threading import time

ITERATIONS = 5_000_000

def worker_adjacent(shared_list, index):

for _ in range(ITERATIONS):

shared_list[index] += 1

def worker_padded(shared_list, index):

# Offset by 16 integers (16 * 8 bytes = 128 bytes > 64-byte cache line)

padded_idx = index * 16 for _ in range(ITERATIONS):

shared_list[padded_idx] += 1

def run_test(target_fn, size):

arr = [0] * size

threads = [threading.Thread(target=target_fn, args=(arr, i)) for i in range(4)] start = time.perf_counter()

for t in threads: t.start() for t in threads: t.join()

return time.perf_counter() - start

if  name  == " main ":

t_adjacent = run_test(worker_adjacent, size=4) t_padded = run_test(worker_padded, size=64)

print(f"Adjacent Indices (False Sharing): {t_adjacent:.4f}s") print(f"Padded Indices (Cache-Aligned): {t_padded:.4f}s") print(f"Slowdown Factor: {t_adjacent / t_padded:.2f}x")

| Memory Layout Configuration | Trial 1 [s] | Trial 2 [s] | Trial 3 [s] | Median Time [s] | Slowdown Penalty Multiplier |
| --- | --- | --- | --- | --- | --- |
| Adjacent Indices (False Sharing Active) | 0.3215 s | 0.3033 | 0.3095 | 0.3095 | — [Baseline] |
| Padded Indices (Cache-Line Aligned) | 0.3124 | 0.3093 | 0.3252 | 03124 | 0.99x |

Student Response & Architectural Justification:

A 64-bit memory reference has a size of 64 bits = 8 bytes. A 64-byte cache line can therefore contain:

64 / 8 = 8 references.

With a stride of 16, the distance between two accessed references is:

16 × 8 = 128 bytes.

Since 128 bytes is greater than one 64-byte cache line, two elements separated by a stride of 16 cannot be located within the same 64-byte cache line. Therefore, the stride-16 layout separates the accessed locations into different cache lines.

Note that CPython integers themselves are larger than 8 bytes because a Python integer is a full Python object. The 8-byte calculation refers specifically to a 64-bit memory reference/pointer, not to the complete size of a CPython int object.

[Q2.1] Calculate the exact memory footprint of a 64-bit integer pointer/reference in CPython. Mathematically prove why an index stride of 16 guarantees that adjacent threads never access memory within the same 64-byte L1 cache line. (8 Points)

Student Response & Architectural Justification:

Assume that array elements 0 and 1 are located on the same 64-byte cache line. Initially, both cores may have the cache line in the Shared state.

When Core 0 writes to index 0, it needs exclusive ownership of the cache line. The coherence protocol invalidates Core 1's copy:

Core 0: Shared → Modified
Core 1: Shared → Invalid

When Core 1 subsequently writes to index 1, it also needs exclusive ownership of the same cache line. Core 0's copy must therefore be invalidated:

Core 0: Modified → Invalid
Core 1: Invalid/Shared → Modified

If Core 0 writes again, ownership moves back to Core 0 and Core 1's copy is invalidated. This repeated transfer of cache-line ownership creates coherence traffic and can cause performance loss. This is the mechanism behind false sharing.

[Q2.2] Trace the MESI state transitions for the shared cache line containing array elements. When Core 0 executes an increment on index 0 and Core 1 subsequently executes an increment on index 1, describe the bus invalidation messages and state shifts (Shared -> Modified -> Invalid). (9 Points)

Student Response & Architectural Justification:

Two industry-standard mechanisms are:

- C++11 alignas(64) — this aligns a variable or object to a 64-byte boundary, helping ensure that frequently modified variables do not share a cache line.

- Java @Contended — this JVM-level annotation adds padding around contended fields so that fields frequently modified by different threads are separated into different cache lines.

These techniques reduce cache-line ownership transfers and therefore reduce false-sharing overhead without requiring application-level synchronization.

[Q2.3] In compiled systems programming, manual array padding wastes memory. Describe two industry-standard compile-time or VM-level mechanisms used to eliminate false sharing (e.g., C++11 'alignas(64)', Java '@Contended', or Go struct field padding). (8 Points)

TASK 3: THE SYNCHRONIZATION TAX & LOCKLESS REDESIGN

Recommended Allocation: 30 Minutes | Task Weight: 25 Points | Concepts: Lock Contention, Race Conditions, Map-Reduce Partitioning

Mutual exclusion primitives (locks/mutexes) prevent data corruption by serializing critical sections. However, coarse-grained locks introduce catastrophic performance penalties: user-space thread suspension, OS kernel context switches (futex system calls), and cache line invalidation across cores competing for the lock variable. In distributed and concurrent systems architecture, the foundational principle is: 'The fastest lock is no lock.'

- EXECUTION PROTOCOL

Create 'task3_sync.py'. Run the benchmark comparing the UnsafeCounter (unprotected shared variable leading to silent data corruption) against the LockedCounter (mutex-protected serialization). Record the corrupted value and the elapsed runtimes.

import threading import time

TOTAL_OPS = 2_000_000 NUM_THREADS = 4

class UnsafeCounter:

def init (self): self.val = 0 def inc(self): self.val += 1

class LockedCounter:

def  init  (self):

self.val = 0

self.lock = threading.Lock() def inc(self):

with self.lock:

self.val += 1

def bench(counter_type):

c = counter_type()

ops_per_thread = TOTAL_OPS // NUM_THREADS def work():

for _ in range(ops_per_thread):

c.inc()

threads = [threading.Thread(target=work) for _ in range(NUM_THREADS)] start = time.perf_counter()

for t in threads: t.start() for t in threads: t.join()

return c.val, time.perf_counter() - start

if  name  == " main ":

val_unsafe, t_unsafe = bench(UnsafeCounter) val_locked, t_locked = bench(LockedCounter)

print(f"Unsafe: Value = {val_unsafe:,} / {TOTAL_OPS:,} | Time: {t_unsafe:.4f}s") print(f"Locked: Value = {val_locked:,} / {TOTAL_OPS:,} | Time: {t_locked:.4f}s") print(f"Contention Cost Multiplier: {t_locked / t_unsafe:.2f}x")

| Counter Architecture | Target Sum | Actual Empirical Sum | Data Corruption Defect Count | Execution Time [s] | Contention Overhead |
| --- | --- | --- | --- | --- | --- |
| Unsafe Accumulator (Race Condition) | 2,000,000 | 2.000.000 | 0 | 0.1515 | 1.00x [Baseline] |
| Locked Accumulator (threading.Lock) | 2,000,000 | 2,000,000 | 0 (Protected) | 0.5203 | 5.64x |

Student Response & Architectural Justification:

In my empirical run, the Unsafe Accumulator produced 2,000,000, exactly matching the target value of 2,000,000. Therefore, the measured corruption defect count was 0.

The theoretical race condition occurs when an increment is implemented as a read-modify-write sequence:

LOAD counter → ADD 1 → STORE counter.

For example, if two threads both LOAD the value 100 before either STORE operation occurs, both threads calculate 101 and both store 101. Two increments have therefore produced only one effective update, resulting in a lost update.

However, this particular Python/CPython experiment did not reproduce the lost-update condition. CPython's execution model and thread scheduling can prevent the race from appearing in a particular run. Therefore, the empirical result should be reported as zero corruption rather than claiming that corruption was observed.

[Q3.1] Analyze your Unsafe Accumulator output. What was your exact corrupted count? Explain the low-level read-modify-write machine code sequence (LOAD, ADD, STORE) that caused lost updates between overlapping thread executions. (7 Points)

- ARCHITECTURAL REDESIGN CHALLENGE: LOCKLESS PARTITIONING

Redesign the accumulation algorithm without using any Mutex locks. Requirements:

- Correctness: The final result must deterministically equal exactly 2,000,000 every single run.

- Performance: Total execution time must be strictly FASTER than LockedCounter by at least 2.0x.

Hint: Use thread-local accumulation buffers (Map-Reduce pattern) where each thread writes exclusively to its own private state, followed by a single reduction stage after threads join.

Student Response & Architectural Justification:

The Lockless Thread-Local Accumulator uses a private counter for each worker thread. Each thread updates its own local value without acquiring a shared lock. After all threads finish, the local values are summed in a final reduction step.

Measured results:

- Locked Accumulator: 0.5202982 s

- Thread-Local Accumulator: 0.0923014 s

- Final sum: 2,000,000

- Defect count: 0

The speedup is:

Speedup = T_locked / T_thread-local

Speedup = 0.5202982 / 0.0923014

Speedup ≈ 5.64×

Therefore, the thread-local map-reduce design achieved approximately 5.64× speedup over the locked accumulator in this experiment.

[Q3.2] Provide your complete Python implementation of the Lockless Thread-Local Accumulator below. Record your measured execution time and calculate the speedup factor achieved over LockedCounter. (11 Points)

Student Response & Architectural Justification:

Eliminating shared mutable state is fundamentally more scalable than optimizing lock primitives because threads do not need to compete for the same shared variable. With a lock, only one thread can enter the critical section at a time, creating serialization and synchronization overhead.

With a thread-local design, each worker updates its own private buffer independently. There is no repeated lock acquisition, lock contention, or ownership transfer for a shared counter. A single reduction operation combines the local results after the parallel work is complete.

Therefore, thread-local accumulation changes the architecture of the program by removing the main source of contention rather than simply trying to make the lock faster.

[Q3.3] Why is eliminating shared mutable state (via thread-local buffers and reduction) fundamentally superior to optimizing lock primitives (e.g., adaptive spinlocks or read-write locks) in high-throughput concurrent systems? (7 Points)

TASK 4: THE MEMORY WALL VS. COMPUTE SATURATION

Recommended Allocation: 25 Minutes | Task Weight: 25 Points | Concepts: Arithmetic Intensity, Roofline Model, DRAM Bus Saturation

The Roofline Model characterizes computational performance by relating Arithmetic Intensity (FLOPs executed per byte of DRAM transferred) to the physical limitations of the host architecture. When a workload has high arithmetic intensity, execution speed is bound by the CPU's ALU/FPU pipelines (Compute-Bound). When arithmetic intensity is low, multiple

cores saturate the off-chip memory bus and memory controllers (Memory-Bound). In memory-bound regimes, adding more parallel workers does not increase throughput; it merely increases memory bus contention and queueing latency.

- EXECUTION PROTOCOL

Create 'task4_roofline.py'. Execute both benchmark suites across 1, 2, and 4 worker processes. The compute-bound suite performs 25,000,000 floating-point operations in registers with zero memory transfers. The memory-bound suite streams 50,000,000 64-bit float arrays (~400 MB per worker) from main memory with minimal math.

import time

import multiprocessing as mp import numpy as np

def compute_heavy(n):

x = 1.0001

for _ in range(n):

x = (x * 1.000001) + 0.00001

return x

def memory_heavy(size):

arr = np.ones(size, dtype=np.float64) arr = arr * 2.0 + 1.0

return arr[0]

def run_suite():

print("=== Compute-Bound Suite (Register Math) ===") for w in [1, 2, 4]:

t0 = time.perf_counter() with mp.Pool(w) as p:

p.map(compute_heavy, [25_000_000] * w)

print(f"Workers: {w} | Execution Time: {time.perf_counter() - t0:.4f}s")

print("\n=== Memory-Bound Suite (DRAM Bandwidth Saturation) ===") for w in [1, 2, 4]:

t0 = time.perf_counter() with mp.Pool(w) as p:

p.map(memory_heavy, [50_000_000] * w)

print(f"Workers: {w} | Execution Time: {time.perf_counter() - t0:.4f}s")

if name == " main ": run_suite()

| Worker Allocation | Compute-Bound Time [s] | Compute Scaling [Tw / T1] | Memory-Bound Time [s] | Memory Degradation [Tw / T1] |
| --- | --- | --- | --- | --- |
| w = 1 Worker (Baseline) |  | 1.00x |  | 1.00x |
| w = 2 Workers |  |  |  |  |

| Worker Allocation | Compute-Bound Time [s] | Compute Scaling [Tw / T1] | Memory-Bound Time [s] | Memory Degradation [Tw / T1] |
| --- | --- | --- | --- | --- |
| w = 4 Workers |  |  |  |  |

Student Response & Architectural Justification:

[Q4.1] Analyze the scaling divergence between the two workloads. Why does running 4 parallel compute tasks complete in roughly the same elapsed time as 1 task, whereas running 4 parallel memory tasks causes total execution time to spike dramatically? (9 Points)

Student Response & Architectural Justification:

[Q4.2] Investigate your host machine's physical memory specifications (e.g., DDR4-3200 vs DDR5-4800, Single-Channel vs Dual-Channel, or Apple Unified LPDDR5). Calculate or look up your laptop's theoretical peak memory bandwidth (in GB/s). Explain how memory bus width limits multi-core scalability. (8 Points)

Student Response & Architectural Justification:

[Q4.3] Suppose you are deploying a machine learning embedding pipeline that transforms high-dimensional feature vectors. Based on your Task 4 findings, would optimizing your code with CUDA GPU kernels or CPU OpenMP pragmas yield greater speedup if your workload is memory-bandwidth saturated? Justify your architectural choice. (8 Points)

EVALUATION RUBRIC & INSTRUCTOR VERIFICATION SIGN-OFF

Total Laboratory Marks: 100 Points | Submission Verification & Academic Audit

| Assessment Category | Evaluation Criteria & Standards | Max Points | Score Awarded |
| --- | --- | --- | --- |
| Task 1: Amdahl Scaling | Complete empirical core sweep table, valid inflection identification, rigorous algebraic derivation of P, and asymptotic limit analysis. | 25 |  |
| Task 2: Cache Coherence | Three-run trial data for adjacent vs padded layouts, mathematical verification of 64-byte cache boundary, and MESI protocol trace. | 25 |  |
| Task 3: Sync & Lockless | Accurate race condition defect capture, lock contention measurement, functioning lockless map-reduce redesign with >2x verified speedup. | 25 |  |
| Task 4: Roofline Analysis | Compute vs Memory scaling data table, identification of DRAM bus saturation, host memory channel analysis, and architectural justification. | 25 |  |
| TOTAL EVALUATION | Comprehensive Silicon Profiling & Architectural Systems Mastery | 100 |  |

| Instructor / TA Walk-Around Verification Check: | Laboratory Submission Audit Timestamp: |
| --- | --- |
| [ ] Task 3 Unsafe Corruption Verified [ ] Task 2 Cache Slowdown Verified [ ] Terminal Logfile 'results_<id>.txt' Uploaded | Examiner Signature: Date: |

APPENDIX: HARDWARE TOPOLOGY QUERY COMMANDS

Use these native OS terminal commands to extract your exact microarchitectural parameters

| Operating System | Command to Retrieve CPU SKU & Core Topology | Command to Inspect Cache & Memory Bus |
| --- | --- | --- |
| Linux (Ubuntu/Debian) | lscpu grep -E 'model name¦cpu cores' /proc/cpuinfo | lscpu -C sudo dmidecode --type memory ¦ grep Speed |
| macOS (Apple Silicon/Intel) | sysctl -n machdep.cpu.brand_string sysctl hw.perflevel0.physicalcpu | sysctl hw.l1dcachesize hw.l2cachesize system_profiler SPMemoryDataType |
| Windows (PowerShell / CMD) | Get-CimInstance Win32_Processor ¦ Select Name, NumberOfCores wmic cpu get name,NumberOfCores,NumberOfLogicalProcessors | Get-CimInstance Win32_CacheMemory wmic memorychip get speed,capacity,devicelocator |

Bonus Task

Copy-paste this code. Do not change the logic. Run it across thread/process counts from 1 to 32, graph your actual execution times, and answer the autopsy questions.

import time

from multiprocessing import Pool import threading

def cpu_work(n):

# Pure CPU crunch count = 0

for i in range(n): count += i * i

return count

# Run both experiments with total_work = 50_000_000 split across T workers # Test T in [1, 2, 4, 8, 16, 32]

Deliverables:

- The Empirical Curve: Plot $T$ (workers) on the X-axis vs. Execution Time on the Y-axis for both threading.Thread and multiprocessing.Pool.

- The Inflection Cliff: Identify the exact number of workers where adding more workers makes the program slower.

- The Architectural Autopsy:

  - Why does Python's threading curve remain virtually flat or get worse, while multiprocessing scales? (Forces them to articulate the GIL).

  - Look up your laptop's CPU model (e.g., Apple M-series, Intel Core i5/i7, AMD Ryzen). How many physical performance cores do you have vs. logical threads?

  - Pinpoint the exact point on your graph where physical core saturation occurs, and explain the performance drop when $T > \text{physical cores}$ using context-switching overhead.